Management of Excess Material

Spec 	180 MUNI

S.P.	#

Add in the following.

180.09		Measurement For Payment

180.09.01 Removal and Management of Non-Reusable Material

Measurement for the removal and management of non-reusable material shall be by the tonne.

180.10		Basis of Payment

Add new item.

180.10.01 Removal and Management of Non-Reusable Material – Item

Payment at the contract price for the above tender item shall include stockpiling, removal, and disposal of unsuitable material from site in accordance with recommendations included as part of Soils Characterization Report attached to this contract and O. Reg. 406/19 On-Site and Excess Soil Management.

Contractor to provide weigh tickets from the disposal facility. Weigh Tickets to conform to GSSS 102