
Adjusting or Rebuilding Structures – Maintenance Holes, Catch Basins, Ditch Inlets, and Valve Chambers
Spec
408 MUNI
SP
##
Scenario A – Surface asphalt to be placed the following construction season (seasonal shutdown)
408.07.01
General 
Add in the following:


Where surface asphalt is being placed the following construction season, the following shall apply:

· Temporary asphalt curb, according to GSSD-610.010, shall be placed behind catch basin frames that are installed to binder asphalt elevation, as directed.  
· Asphalt ramping shall be placed around structure frames that are installed to final asphalt elevation, as directed.

· Adjustment of new structure frame & grate/cover from binder asphalt elevation to surface asphalt elevation shall be paid under the item excluding supply of frame & grate/cover.
· If sidewalk is abutting curb, temporary asphalt sidewalk to be placed along the temporary asphalt curb
All maintenance holes shall have frames and covers installed to binder asphalt elevation prior to winter shutdown.

Auto-Stable Self-Level Frame and Cover to be installed where it is determined that there is sufficient space between finished grade and roof of structure to allow for proper installation.  Where sufficient space is not available, standard frame and cover, as per OPSD 401.010, to be installed.

Prior to surface asphalt being placed, all structures shall be adjusted to finished grade.  The temporary asphalt curb behind the Catch basin grate shall be removed and the concrete curb installed.
408.09

Measurement for Payment


408.09.01
Actual Measurement


Add in the following:

Payment will be made of one (1) each per adjustment. Adjustment of new structure frame & grate/cover from binder asphalt elevation to surface asphalt elevation shall be paid under the item excluding supply of frame & grate/cover.
408.10
Basis of Payment 
408.10.01

Add in the following:
Adjusting Maintenance Holes and Valve Chambers – Auto-Stable Self Level frame and cover - Item


Payment at the Contract price shall include the following: 

· Removal and disposal of existing moduloc and sawcutting, removal and disposal of existing asphalt around the structure;
· Payment will be made of one (1) each for each structure adjusted, 75% to be paid at placement to binder asphalt elevation, remainder to be paid upon adjustment to surface asphalt. 
Adjusting Maintenance Holes and Valve Chambers – Standard Frame and Cover - Item
Payment at the Contract price shall include the following: 

· Removal and disposal of existing moduloc and sawcutting, removal and disposal of existing asphalt around the structure;

· Placement of asphalt patching to binder asphalt elevation;

· Payment will be made of one (1) each per adjustment. Adjustment of structures from binder asphalt elevation to surface asphalt elevation to be paid under separate item without payment for new frame & cover. 
Adjusting Catch Basins and Double Catch Basins - Item
· Removal and disposal of existing moduloc and sawcutting, removal and disposal of existing asphalt around the structure;

· Placement of asphalt patching to binder asphalt elevation;

· Placement and removal of asphalt ramping to structures placed at final asphalt elevation. 

· Placement of temporary asphalt curb and sidewalk behind structure

· Removal of temporary asphalt curb

· Payment will be made of one (1) each per adjustment. Adjustment of structures from binder asphalt elevation to surface asphalt elevation to be paid under separate item without payment for new frame & cover. 
Designer Note: 

· Designer to determine number of structures that will require a second adjustment not including frame and grate/cover.
· For quantity estimate designer to assume all CB/DCB’s placed at intersections and at low points to be installed initially at binder asphalt elevations.  All other CB/DCBs to be installed at final asphalt elevation.
· Auto Stable frame & covers do not require a second adjustment item.  

· Standard frame & covers require a second adjustment item 

· Resetting of existing frame and grates to occur in specific instances where new frame and grates/covers are not available.  Example: Wide gutter inlet on cast in place structures.
· Separate payment for temporary asphalt sidewalks to be determined on a contract-by-contract basis, dependent upon total quantities required and number of adjustments requiring asphalt sidewalks relevant to the total numbers of structures adjusted. 
Scenario B – Surface asphalt to be placed in the same construction season with phasing where road will be open to traffic on binder asphalt (1 month +).  Primarily multi-lane roads
408.07.01
General 

Add in the following:


Where surface asphalt is being placed later in the same construction season, the following shall apply:

· When structure grates are installed to final asphalt elevation, asphalt ramping shall be placed around the structure.

· When Frame and Cover is installed, the initial installation shall be to binder asphalt elevation.
· Adjustment of new structure frame &grate/cover from binder asphalt elevation to surface asphalt elevation shall be paid under the item excluding supply of frame & grate/cover
Auto-Stable Self-Level Frame and Cover to be installed where it is determined that there is at least 300 mm below finished grade to allow for proper installation.  If 300 mm is not available, standard frame and cover, as per OPSD 401.010, to be installed.

Prior to surface asphalt being placed, all structures shall be adjusted to finished grade.  
408.09

Measurement for Payment


408.09.01
Actual Measurement


Add in the following:

Payment will be made of one (1) each per adjustment. Adjustment of new structure frame & grate/cover from binder asphalt elevation to surface asphalt elevation shall be paid under the item excluding supply of frame & grate/cover.
408.10

Basis of Payment 

408.10.01

Add in the following:
Adjusting Maintenance Holes and Valve Chambers – Auto-Stable Self Level frame and cover - Item
Payment at the Contract price shall include the following: 

· Removal and disposal of existing moduloc and sawcutting, removal and disposal of existing asphalt around the structure;

· Payment will be made of one (1) each for each structure adjusted, 75% to be paid at placement to binder asphalt elevation, remainder to be paid upon adjustment to surface asphalt. 
Adjusting Maintenance Holes and Valve Chambers – Standard Frame and Cover - Item
Payment at the Contract price shall include the following: 

· Removal and disposal of existing moduloc and sawcutting, removal and disposal of existing asphalt around the structure;

· Placement of asphalt patching to binder asphalt elevation;

· Payment will be made of one (1) each per adjustment. Adjustment of structures from binder asphalt elevation to surface asphalt elevation to be paid under separate item without payment for new frame & cover. 
Adjusting Catch Basins and Double Catch Basins – Item
· Removal and disposal of existing moduloc and sawcutting, removal and disposal of existing asphalt around the structure;

· Placement of asphalt patching to binder asphalt elevation;

· Placement and removal of asphalt ramping to structures placed at final asphalt elevation. 

· Payment will be made of one (1) each per adjustment. Adjustment of structures from binder asphalt elevation to surface asphalt elevation to be paid under separate item without payment for new frame & cover. 
Designer Note: 

· Designer to determine number of structures that will require a second adjustment not including frame and grate/cover.

· For quantity estimate designer to assume all CB/DCB’s placed at intersections and at low points to be installed initially at binder asphalt elevations.  All other CB/DCBs to be installed at final asphalt elevation.

· Auto Stable frame & covers do not require a second adjustment item.  

· Standard frame & covers require a second adjustment item 

· Resetting of existing frame and grates to occur in specific instances where new frame and grates/covers are not available.  Example: Wide gutter inlet on cast in place structures.

Scenario C – Surface asphalt to be placed immediately following upper binder asphalt
408.07.01
General 

Add in the following:


Auto-Stable Self-Level Frame and Cover to be installed where it is determined that there is at least 300 mm below finished grade to allow for proper installation.  If 300 mm is not available, standard frame and cover, as per OPSD 401.010, to be installed.

408.10
Basis of Payment 

408.10.01 

Add in the following

Adjusting Maintenance Holes and Valve Chambers – Auto-Stable Self Level frame and cover - Item
Payment at the Contract price shall include the following: 

· Removal and disposal of existing moduloc and sawcutting, removal and disposal of existing asphalt around the structure;

· Adjusting Maintenance Holes and Valve Chambers – Standard Frame and Cover - Item
Payment at the Contract price shall include the following: 

· Removal and disposal of existing moduloc and sawcutting, removal and disposal of existing asphalt around the structure;

· Adjusting Catch Basins and Double Catch Basins – Item
· Removal and disposal of existing moduloc and sawcutting, removal and disposal of existing asphalt around the structure;

408 MUNI – Adjusting or Rebuilding Structures – Maintenance Holes
January 2023

Catch Basins, Ditch Inlets and Valve Chambers


