Asphalt Sidewalk, Driveway, Boulevard and Sidewalk Resurfacing

Spec	311 MUNI

S.P.	#

311.02		References

Add in the following:

OPSS 310 MUNI	Hot Mix Asphalt

311.07 	Construction

311.07.01 	General

Add in the following:

This specification applies to the placement of asphalt for area such as driveways, parking lots, walkways, boulevards, swales, and patching at the depths indicated below:

Boulevards, driveways, parking lots, swales:  – Surface HL3 (PG 52-34) – 50 mm.

Commercial Entrances:	– Surface HL3 (PG 52-34) – 40 mm.

– Binder HL8 (PG 52-34) – 50 mm or match existing depth, whichever is greater.

Local Roads: 	– Surface HL3 (PG 52-34) – 40 mm.

- Binder HL8 (PG 52-34) – 50 mm.

Arterial/Collector Roads:	– Surface HL3 (PG 64-34) – 40 mm.

– Heavy Duty Binder Course (PG 64-34) – 50 mm x 2 Lifts.

* items not required for the contract may be removed

The placement of asphalt within boulevards shall be in a continuous operation for full width and length with no breaks in the paving operation between entrances or side streets.

311.09 	Measurement for Payment

311.09.01 	Actual Measurement

Add in the following:

311.09.01.01 	Placing Hot Mix Asphalt – Item

Measurement for supply and placement of asphalt at the depths indicated above shall be by area in square metres.

311.10 	Basis of Payment

Add in the following:

The method used to place the asphalt shall be determined by the Contractor (i.e.: small spreader or hand placement). If at the direction of the General Manager, a different thickness of asphalt is required, the unit price bid per square metre will be pro-rated to reflect this change.