Installation of Interlocking Concrete Pavers

Spec 	355 MUNI

S.P. 	#

355.02	References

Add in the following:

OPSS 510 MUNI Removal

355.07.06 	Laying Pattern

Add in the following:

When restoring or adding to existing concrete pavers, the Contractor shall relay the pavers and borders to match the existing pattern.

355.07.10 	Removal of Unacceptable Concrete Pavers

Add in the following:

The Contractor shall be responsible to replace any shortages that may occur due to damage done during removals and/or recutting. Any replacement pavers shall be similar in style and colour to the existing pavers.

355.07.11 	Management of Excess Material

Add in the following:

Any excess interlocking Concrete Pavers shall be stockpiled on site or disposed of at the direction of the property owner.

Add in the following:

355.07.12	Removal and Salvage of Interlocking Concrete Pavers

Where reinstatement of interlocking concrete pavers is specified, work shall include the removal, salvaging, stockpiling and protection of interlocking concrete pavers.

355.10	Basis of Payment

Add in the following:

355.10.02	Reinstatement of Interlocking Concrete Pavers - Item

Payment at the Contract price for the above item shall be full compensation for all labour, equipment, and material to do the work.

For the Contract price bid, the Contractor shall remove, stockpile, and protect the existing paving stones and edging, regrade the driveway with suitable crusher dust, and relay the pavers and borders.