Restoration

*Designer to select contract type

Full Road Reconstruction:

Restoration shall be as follows and paid for under the applicable contract items.

Pipe restoration contract:

Payment at the contract price for all pipe work and structures shall include removal and disposal of granular road base, subbase material and complete restoration of trenches as specified below with the exception of asphalt, concrete curbs, sidewalks, and sod restorations where specified elsewhere in the Contract.

Pipe restoration contract with contaminated soil:

Payment at the contract price for all pipe work and structures shall include complete restoration of trenches as specified below with the exception of asphalt, concrete curbs, sidewalks, and sod restorations where specified elsewhere in the Contract.

* Designer to specify restoration conditions of contract

- Roadways

Collector/Arterials Road

- 40 mm Surface Course Asphalt HL3 (PG64-34)

- Tack Coat

- 50 mm Heavy Duty Binder Course Asphalt (PG64-34)

- Tack Coat

- 50 mm Heavy Duty Binder Course Asphalt (PG64-34)

- 150 mm Granular ‘A’

- 600 mm Granular ‘B’ Type II

Local Roads

- 40 mm Surface Course Asphalt HL3 (PG52-34)

- Tack Coat

- 50 mm Binder Course Asphalt HL8 (PG52-34)

- 150 mm Granular ‘A’

- 600 mm Granular ‘B’ Type II

* Site conditions may determine different pavement structures/requirements

- Driveways and Entrances

- Commercial / Institutional / Industrial entrances

- 40 mm HL3 Surface Asphalt (PG 52-34)

- Tack Coat

- 50 mm HL8 Binder Asphalt (PG 52-34)

- 150 mm Granular ‘A’

- 300 mm Granular ‘B’ Type II (min. or match existing which ever is greater)

* Designer to confirm entrance requirements

- Residential

As per GSSS 492, unless otherwise specified

* When surface asphalt is deferred on rural road sections, designer to specify temporary driveway treatments

- Driveway Curb and Retaining Walls < 0.5m (Precast or Poured Concrete, Moduloc Block, Native Stone, and Timber)

Where possible, remove, salvage and reinstate all existing retaining walls and driveway curbs to original conditions. This work shall be included in the item required for the work. When directed by the General Manager, new retaining walls shall be constructed and paid under the appropriate item.

* To be included in restoration contracts only as required

- Grassed Areas

The restoration of grassed areas shall be in accordance with GSSS 802 and paid for under the appropriate items.

* To be included in restoration contracts only where sod is to be included in the pipe/item price.

- Reinstatement of Existing Landscaping Features

All existing landscape features including, but not necessarily limited to; garden and driveway edging, precast/wood retaining walls under 0.15m, concrete steps, patios, etc. (excluding trees within the City Right of Way), disturbed by the work shall be reinstated to existing condition as directed.  All surplus landscape features shall be salvaged and returned to the property owner as directed.

When the Contract does not contain a separate item for the restoration of a particular landscape feature, restoration shall be deemed inclusive in the price of the items requiring the work.

The Contractor shall confirm any removals on private property with the General Manager.