###### Test Digs – Hydro Excavation

###### Spec	NIL

###### S.P.	#

Scope

This specification covers the requirements for the completion and restoration of test digs for purposes of verifying depth, locations and/or potential conflicts of underground infrastructure in advance of the work or as required by the General Manager.

References:

OPSS 206	Grading

OPSS 401	Trenching, Backfilling, and Compacting

OPSS 492	Site Restoration following Installation of Pipelines, Utilities and Associated Structures

Construction

Test Digs shall be completed via hydro excavation equipment.

This item shall apply to investigations, exposure or physical located that are a requirement of applicable utilities. This item may also be used at the discretion of the General Manager for locating other underground infrastructure where test digs via excavation are deemed unsuitable.

*Designer consideration: Designer to review requirements for each utility. If 3rd party requirements are required (i.e. Enbridge). Their 3rd party requirements are to be placed in Division 5 and referenced here.

Measurement for Payment

Measurement shall be by each for the number of test digs performed.

Each test dig shall measure up to a maximum horizontal dimension of 1.0 metres by 1.0 metres.

Each test dig shall be to a maximum depth of 2.0 metres.

Basis of Payment

Payment at the contract price for the tender items shall be full compensation for all labour, equipment, and material to do the work, including any coordination with utility companies.

Restoration of test digs shall be included in the unit price bid up to surface materials.  Concrete, asphalt, sod, and landscaping finishes to be paid for separately under the appropriate contract item.