###### Test Digs

###### Spec	NIL

###### S.P.	#

Scope

This specification covers the requirements for the completion and restoration of test digs for purposes of verifying depth, locations and/or potential conflicts of underground infrastructure in advance of the work or as required by the General Manager.

References:

OPSS 206	Grading

OPSS 401	Trenching, Backfilling, and Compacting

OPSS 492	Site Restoration following Installation of Pipelines, Utilities and Associated Structures

Construction

The method and equipment used to perform the test digs shall be determined by the Contractor.

This item shall not apply to investigations, exposure or physical locates that may be a requirement of applicable utilities.

Measurement for Payment

Measurement shall be by each for the number of test digs performed.

Each test dig shall measure up to a maximum horizontal dimension of 2.0 metres by 2.0 metres.

Each test dig shall be to a maximum depth of 3.0 metres.

*Designer consideration: Designer to review potential test dig depths, if test digs to be deeper than 3.0 metres, potential for additional item for depths greater than 3.0m or update SP.

Basis of Payment

Payment at the contract price for the tender items shall be full compensation for all labour, equipment, and material to do the work, including any coordination with utility companies.

Restoration of test digs shall be included in the unit price bid up to surface materials.  Concrete, asphalt, sod, and landscaping finishes to be paid for separately under the appropriate contract item.

*Designer considerations: Designer to review water and sanitary service Y-connections and determine potential test dig locations on private property.