Removals

Spec	510 MUNI

S.P.	#

510.07.03	Drainage Work

510.07.03.02 	Removal of Curb and Gutter

ADD in the following:

The work shall include sawcutting all edges.

510.07.03.09	Abandonment of Pipes and Culverts

Add in the following:

Where sections of pipes are identified to be abandoned and grouted, they shall be filled with grout.

Access points shall be provided as required to allow for confirmation that the pipe has been completely filled.

510.07.06 Pavement Work

510.07.06.01 General

Add in the following:

This item shall include hauling and stockpiling of the RAP material to (NAME) Depot.

* Designer Note: This item is only to be included If millings are to be delivered to a depot.

510.07.06.02 Cutting Existing Pavement

Add in the following:

Saw cuts on roadway shall be as per GSSD 525.010 where practical.

510.07.06.03 Removal of Pavement, Treated Base, and Concrete Base

Add in the following:

This item shall be intended for the full depth removal of existing asphalt in boulevards, driveways, and the roadway asphalt at the curb cuts

* Designer Note: This item is intended for contracts where the roadway is being removed using partial depth asphalt removal.

510.07.08	Right-of-Way Work

510.07.08.03	Removal of Concrete Sidewalk

ADD in the following:

This work shall include sawcutting all edges at the nearest joint

510.09 MEASUREMENT FOR PAYMENT

510.09.01 Actual Measurement

ADD in the Following:

510.09.01.18 Removal of Asphalt Pavement, Partial-Depth

Measurement of removal of partial-depth asphalt pavement shall be by area in square metres to a depth of ##mm.

510.09.01.25 Cutting Existing Pavement

Measurement of the cutting of existing pavement shall be by linear meter at the width of the equipment being used.

510.10 	Basis of Payment

510.10.01

Delete paragraph 6 and add in the following:

Where the removal, disposal and capping/abandoning of pipe subdrain, pipes, culverts, force mains or water main appurtenances, is required as they interfere with the execution of the work, the contract price for the items requiring the removal of the pipe subdrain, pipes, culverts, force mains or water mains shall include full compensation for all labour, Equipment and Materials required to do the work described in this specification.

Designer Considerations

The method or intent of removal needs to be identified. “By milling” “For driveways, boulevards etc.” and needs to be identified in the SUP to provide further clarity